[image: image1.png]



[image: image66.jpg]



Featurephone 175 Manual

INSTALLATION AND OPERATING INSTRUCTIONS

(Model number 175)

Featurephone 175

[image: image68.png]



1.
Handset

2.
Menu-keys

3.
Backlight display

4.
Function-keys

5.
Speaker

6.
Handset cord

7.
Numerical keys

8.
CryptoCard reader

9.
CD drive

10.
Recorder-keys

11.
Speakerphone key

12.
Volume-adjustment

Connections

[image: image2.png]



1.
Power supply

2.
RS 232 serial port

3.
Start/Stop switch

4.
Loudspeaker

5.
Microphone

6.
Phone line Out

7.
Phone line In

Recorder-keys

The recorder keys have specific functions in different situations although the operation is always very similar. During recording and playback the keys have the standard recorder key functions as explained below.

[image: image3.png]< I




Record           Backward        Stop          Pause/Play    Forward          Skip

The [image: image4.png]


 and [image: image5.png]


 keys can be used to scroll through numbers in the telephone book or through a settings menu. If no recording or playback is taking place these keys always have an Up/Down or Left/Right function. The  [image: image6.png]


 key will always exit the current operation.

Function-keys

        [image: image7.png]



from left to right:

1.
Hook control

2.
Playback calls

3.
Flash/History

4.
Playback Microphone recordings

5.
Telephone book

6.
Playback Messages

7.
Disk menu

8.
Configuration

Alfa-numerical functions

[image: image8.png]Numerical keys | Function

Press the relevant key repeatedly or hold down:

> o
z —_
u
un
a
in

i F [:1 = J
DEF
L ) I ] <
GHI ,

i u] i Ep
KL .

MNO
i1 g i £ 8

-
o
2
()

-4

7 E i = 5
TUV T N N "
| 1 11 = &

WXYZ

L

# i 3 L 1

Toggle uppercase and lowercase

Recorder keys

HH S ©
f

v
™

Forward
Backward





Care and Maintenance

[image: image9.png]



Keep the FeaturePhone 175 dry. If it gets wet, wipe it dry immediately Liquids might contain minerals that can corrode the electronic circuits.

[image: image10.png]0°to 40°}




Use and store the FeaturePhone 175 only in normal temperature environments. Temperature extremes can shorten the life of electronicdevices and distort or melt plastic parts.

[image: image11.png]



Keep the FeaturePhone away from excessive dust and dirt that can cause premature waer of parts.

[image: image12.png]



Do not use harsh chemicals, cleaning solvents or strong detergents to clean the FeaturePhone 175. 

Table of contents

121
Getting started


121.1
Out of the box


131.2
Connecting the Handset


141.3
Connecting the power supply


141.4
Connecting to the telephone line


141.5
Setting the FeaturePhone 175 up for use


141.5.1
Set the clock


151.5.2
Select the language


172
Using the FeaturePhone 175


172.1
Handset / Headset / Speakerphone


182.2
Initialising CD


182.3
Start a call


182.4
Accept a call


192.5
End a call


192.6
Adjust the Volume


203
Operating the FeaturePhone 175


203.1
Frequently used key indicators


203.2
Save Changes


214
Recording calls


214.1
Automatic recording


214.2
Copy recordings to CD


214.3
Making a CD suitable for the PC


214.4
Notification


224.5
Recording quality


224.6
Fax Recording


224.7
Recording from the microphone


234.8
Microphone mode


234.9
Automatic Gain Control


255
Search and play recordings


255.1
Search recordings


255.2
Search according to date


265.3
Search according to code


286
Answering machine


286.1
Turn the answering machine On


286.2
Record welcome message


296.3
New Messages.


296.4
Playback messages


296.5
Remove messages


317
Operating the CD recorder


317.1
Free time left on CD


317.2
Copy the memory to CD


317.3
Copy a recording from CD to memory


327.4
Create CD directory


337.5
Play audio CD


347.6
Erase CD Re-Writable


347.7
Erase memory


357.8
Total recordings


357.9
Software update


388
The Telephone book


388.1
Add a contact


388.2
Making a call from the telephone book


388.3
Search a contact


398.4
Edit a contact


398.5
Remove a contact


398.6
Call History


419
Using the CryptoCard


419.1
Insert the CryptoCard into the FeaturePhone 175


419.2
The CryptoCard and its PIN and PUC code


419.3
Making an encrypted recording


429.4
Playing an encrypted recording


4410
Settings


4510.1.1
Enter General Settings


4510.1.2
Password protection


4610.1.3
CryptoCard


4710.1.4
Clock


4710.1.5
Language


4810.1.6
Reset all values


4910.2
Telephone Settings


4910.2.1
Enter the Telephone Settings


5010.2.2
Caller ID


5010.2.3
Put on hold


5010.2.4
Flash time


5110.2.5
Wait after dial


5110.2.6
Ring settings


5210.3
Recorder Settings


5210.3.1
Enter the Recording Settings


5210.3.2
Automatic recording


5210.3.3
Automatic CD Copy


5410.3.4
Notification


5410.3.5
Ask Store/Delete


5410.3.6
Beep after store


5510.3.7
Minimum call time


5510.3.8
Recording Quality


5510.3.9
Fax Recording


5510.3.10
Microphone mode


5610.3.11
Microphone amplification


5610.3.12
Automatic gain control


5610.3.13
Humm filter


5710.3.14
Footswitch


5710.4
Answering machine Settings


5710.4.1
Enter the Answering machine Settings


5810.4.2
Turn Answering machine on


5810.4.3
Welcome message


5810.4.4
Rings to answer on


6010.4.5
Rings always answer


6010.4.6
Speaker after answer


6010.4.7
K-break detection


6110.4.8
Maximum message time


6110.4.9
Silence period


6211
Using the PC software


6211.1
Install the software


6211.2
Start


6211.3
View recordings on CD


6311.4
Create an archive on Hard disk


6311.5
Recycle bin


6411.6
Export recordings


6411.7
On Line software update


6411.8
Search recordings


6511.9
Software Settings


6812
Accessories


6913
Conformation with directive 1999/5/EC and Class 1 of the R&TTE


7014
Guarantee Conditions


7115
Legality of Recording


7316
Index




.

Getting started

In this Chapter the connections XE "connections"  of the FeaturePhone 175 are described.

1.1 Out of the box

In the box of the FeaturePhone 175 you will find:

FeaturePhone 175

handset XE "Handset" 
curled handset cable with two RJ 10 connectors

7,5 volt AC power adapter

flat telephone cable with two RJ 11 connectors

CD recordable

software CD

manual

Connecting the Handset

The handset is connected to the FeaturePhone 175 with the supplied curled cable with two RJ 10 connectors.  

[image: image13.png]COrree




· Connect one side of the curled cable to the connector at the bottom of the handset.

[image: image14.png]



· Connect the other side of the curled cable to the handset connector at the bottom of the FeaturePhone 175.

Connecting the power supply

The FeaturePhone 175 is powered through a wall socket power adapter.

[image: image15.png]vidicode

Fower hone 175 #5232 switch 5





· First connect the AC power adapter XE "Power adapter"  to the wall socket. 

· Then connect the mini jack of the adapter to the Power connector on the back side of the FeaturePhone 175.

1.2 Connecting to the telephone line

Connect the FeaturePhone 175 to the telephone line XE "Telephone line"  with the supplied flat cable with two RJ11 connectors.

[image: image16.png]



· Connect one end of the telephone cable to the telephone line wall socket. (For UK only use the supplied BT telephone line adapter)

· Connect the other side of the telephone cable to the Line connector at the back of the FeaturePhone 175.

1.3 Setting the FeaturePhone 175 up for use

To gonfigure the FeaturePhone 175 for use the Clock needs to be set and the Language needs to be selected.  These settings are part of the General Settings menu.  Setting the Clock and selecting the Language is described in the next two paragraphs.

1.3.1 Set the clock XE "Clock" 
Set the clock as follows:

Press [image: image17.png]


Configuration.
General Settings

NEXT  MENU           STOP

Press MENU  to enter the General Settings menu.

Press NEXT twice

The display will show:

 General Settings 

Clock: Mon 12-06-03 10:31           NEXT          CHANGE BACK

Press CHANGE to set the clock

 General Settings 

Clock: ▓on 12-06-03 10:31           STORE   <<    >>   CANCEL

Press keys 1 - 7 to change the day beginning with 1 for Sunday

 General Settings 

Clock: Mon ▓2-06-03 10:31           STORE   <<    >>   CANCEL

Enter the date (format depending on selected language)

 General Settings 

Clock: Mon 12-06-03 ▓0:31           STORE   <<    >>   CANCEL

Enter the time

 General Settings 

Clock: ▓on 12-06-03 10:31           STORE   <<    >>   CANCEL

Press STORE to save the time.

Other options:


CANCEL to return to previous set time.


<< and >> to move the cursor.

 General Settings

Clock: Mon 12-06-03 10:31           NEXT          CHANGE BACK

Press NEXT to continue with language selection.

1.3.2 Select the language XE "Language" 
Select the language as follows:

Enter the General Settings menu and scroll through the menu or continue from the previous paragraph by pressing NEXT: 

The display will show:

 General Settings

Language:     English  NEXT          CHANGE BACK

Press CHANGE to scroll through the available options.

Together with the Language the date format can change also:

Standard  date format is : DD-MM-YY  

When set to American the format is changed to : MM-DD-YY

Using the FeaturePhone 175

The FeaturePhone 175 is a special telephone aimed at the automatic recording of telephone conversations. For this purpose the FeaturePhone 175 is equipped with a CD recorder. The FeaturePhone 175 can be connected to any regular analog telephone line and is used as a regular telephone. It has the ability to automatically record telephone calls as they are being made. Telephone calls are recorded in flash memory during the call. After the calls have ended the recordings are copied to CD. Both CD recordable and CD rewritable’s can be used in the FeaturePhone 175.

The FeaturePhone 175 can also be used as headset telephone. The FeaturePhone 175 has a standard headset interface to which the optional headset can be connected. If desired the handset can be disabled when using the headset. The FeaturePhone 175 also features a speakerphone function. 

Other features of the FeaturePhone 175 include the phonebook and microphone recording ability. Recordings can be encrypted using the CryptoCard option. Encrypted recordings can only be played using the same or a matching card to the card the recording was made with. The telephone can even be disabled without a CryptoCard present.

1.4 Handset / Headset / Speakerphone

The FeaturePhone 175 features a handset and speakerphone XE "Speakerphone" . Both the handset and the speakerphone key control Hook contact:. The Handset controls Hook contact by a magnetic proximity detector mounted inside the device case below the rest for the microphone side of the handset. Inside the handset a small magnet is mounted that allows the FeaturePhone 175 to detect whether the handset is lifted frrom the phone or put down again. 

The handset control takes precedence over the speakerphone. If the handset is Off the hook, press the Speakerphone key to transfer the call to the speakerphone. The display will show “Speakerphone ON”. Press speakerphone key again to transfer the call back to the handset. The display will show “Speakerphone OFF”. If the call is on speakerphone and the handset is off the hook then putting down the handset will end the call.

An optional headset XE "Headset"  can be connected to the FeaturePhone 175. The headset and handset can be used interchangeably. When the headset is used the Hook control function key can be used for On and Off hook.

Initialising CD

During use of the FeaturePhone 175 there are various moments where the CD drive has to be initialised XE "Initialise CD"  before it can read and write.

During initialisation the display will show:

Initialising CD

At this point the FeaturePhone 175 cannot be used for any other functions. The initialisation of the CD may take upto 30 seconds.

At other moments when the CD is used the display will show:

One moment please!

Also at this point the FeaturePhone 175 cannot be used for any other operations.

1.5 Start a call

There are two ways to start a call XE "Start a call" .

Pick up the handset and wait for the dial tone. The display will show:

Number:

Dial the number and wait for the remote phone to ring.

It is also possible to use the FeaturePhone 175 using mobile phone style:

First enter the number. The display will show:

Number: 0018563234543

STORE         CLEAR  STOP

Then pick up the handset

The display will show:

Dial: 0018563234543

The number is dialed. 

Instead of picking up the handset the speakerphone key can be used as well.

In case a headset is connected to the FeaturePhone 175 the Hook control function key can be used as well.

It is also possible to start a call from the Telephone book. See § 8.2
1.6 Accept a call XE "Accept a call" 
When the phone is ringing the display will show:

 Monday   07-07-03  11:57

Telephone ringing      1x

The FeaturePhone 175 will keep track of the number of rings that have passed and show it in the display.

Pick up the handset to accept the call.

The speakerphone key can also be used to accept the call.

In case a headset is connected to the FeaturePhone 175 the Hook control function key [image: image18.png]


 can also be used to accept the call.

1.7 End a call XE "End a call" 
A call is ended by placing the handset back on its rest or by pressing the speakerphone key when the speakerphone was used. 

In case a headset is connected to the FeaturePhone 175 the Hook control function key can be used as well.

1.8 Adjust the Volume

The volume XE "volume"  of the FeaturePhone 175 can be adjusted easily.

Use the + and - keys below the speaker to adjust the volume. Both for speakerphone and handset use the volume can be set separately.

The FeaturePhone 175 will keep the speaker volume settings automatically.

[image: image19.png]



2 Operating the FeaturePhone 175

The FeaturePhone 175 has a common menu driven user interface XE "User interface" . Most function and operations are initiated by pressing a function key followed by a sequence of display determined menu keys. The use of function keys and menu keys is indicated as follows:

Function keys

The most frequently used functions of the FeaturePhone 175 have been grouped in the function keys XE "Function keys" . Throughout the entire manual figures representing the function keys are used to indicate the function key corresponding to the described function. Pressing one of the function keys will always result in a display where the various options are assigned to the four menu keys. 

Menu keys

After pressing a function key the available options are assigned to the four menu keys XE "Menu keys" . The operation of the menu keys is determined by the text in the display right above the corresponding key. Throughout the entire manual figures representing the display withor without the menu keys are used to show the required action corresponding to the described feature.

Example:

General Settings

NEXT  MENU           STOP

       █████  ▓▓▓▓▓  █████  █████
In this example you are instructed to press the menu key below the text MENU in order to open the General setting menu.

2.1 Frequently used key indicators XE "Key indicators" 
The following menu key functions are consistently used throughout the manual.

MENU in the display indicates the presence of an underlying menu

Press MENU to open the underlying menu

NEXT in the display indicates the presence of more menu items

Press NEXT to jump to the following menu item

BACK in the display indicates the presence of an embracing menu

Press BACK to return to the embracing menu 

STOP in the display indicates the absence of an embracing menu

Press STOP to return to operating mode of the FeaturePhone 175 

CHANGE will toggle the item in the display between "On" and "Off"

In some cases CHANGE is used to increase or decrease a value.

2.2 Save Changes

After making settings the user is forced to press YES or NO before returning to operation mode. Press STOP in the menu and the display will show:

Save Changes?

YES    NO          CANCEL

YES returns the FeaturePhone 175 to normal use with changed settings 

NO returns the FeaturePhone to normal use without changed settings

CANCEL returns to the settings

3 Recording calls XE "Recording calls" 
By default factory settings all telephone calls are recorded. Recordings are primarily stored in flash memory. If a call is ended the recording is automatically copied to CD. The recording also remains in memory.

Automatic copying of recordings to CD can be disabled in the Recording Settings menu, see § 4.1. When Automatic copying is disabled recordings can be copied to CD manually as described in § 7.2.

When recordings have been copied to CD they can be played back by the FeaturePhone 175 but not yet on a PC. For the PC to be able to play the recordings the CD has to have a PC compliant directory.  The CD can have an open or a closed directory. See § 7.3 

3.1 Automatic recording XE "Automatic recording" 
By default all calls are recorded automatically.

Automatic recording can be disabled in the Recording Settings menu, see § 10.3.2. When Automatic recording is disabled recording can be started manually using the recorder keys.

There is one exception to Automatic recording: When the use of the announcement message has been enabled recording of outgoing calls need to be started manually. 

For incoming calls the recording announcement message is played when the phone is picked up. 

For outgoing calls it is not possible to determine what the correct moment to play the message is. It has therefore been made that the recording can only be started manually and the message is played directly after the recording is started.

The display will show START when the announcement message has been selected in the menu.

3.2 Copy recordings to CD XE "Copy recordings to CD" 
During a call the recording is stored in the memory of the FeaturePhone 175. Once the call has ended the recording is copied to CD. The automatic copying of recordings from memory to CD can be turned ON and OFF in the Recording Settings menu. 

When Automatic copy to CD has been disabled the unwanted recordings can be removed from the memory. When all undesired recordings have been removed the remaining recordings can be copied to CD. The copying of recordings from memory to CD is described in § 7.2.

3.3 Making a CD XE "Making a CD"  suitable for the PC

A CD can only be read by a PC when the CD contains a directory of all the files that are stored on the CD. For the FeaturePhone 175 this is not the case. The FeaturePhone 175 can read the recordings on the CD without the directory. 

Creating a directory on the CD is described in § 7.3. 

A CD can have more then one directory made. 

The advantage of having a directory is that the CD can be read by the PC and the search for a recording by the FeaturePhone 175 is performed much faster, certainly when there are a large number of recordings on the CD.

The downside to creating a directory on the CD is that it takes up 16 Mb of storage space on the CD which is taken away from the storage capacity of the CD. 

3.4 Notification XE "Notification" 
In certain countries or regions the recording of the telephone call has to be announced to the remote caller. There are two ways to let the remote caller know the call is being recorded

Notification tone. The notification tone is a regularly returning beep. The beep can be heard both by the local and the remote caller. The FeaturePhone 175 features a smart notification tone where the beep is sounded only when a silence occurs during the call. This way you can always be sure the remote caller will hear the beep.

Notification message: The notification message is a spoken message that can notify the callers the call is being recorded.  The message has to be recorded first. 

The sounding of the notification message is different for incoming and outgoing calls.

For incoming calls the message is sounded when the phone is being picked up. For outgoing calls the moment of playing the message is determined manually. The reason being that the FeaturePhone 175 cannot determine when the remote side picks up the phone. Even when Automatic recording is enabled the recording of outgoing calls has to be started manually when the notification message is enabled. The recording is started by pressing START in the display. The notification message is played when the recording is started.

The notification tone and message can be enabled separately and together. See § 10.3.4 for details.

3.5 Recording quality XE "Recording quality" 
The recording quality refers to the format the of file that the recording is stored in. The file format is determined by the way the recording is compressed. The compression in turn determines the file size and the quality of the recording.  A high compression rate generally has a poorer quality.

The FeaturePhone 175 supports 5 different formats. The table below shows the relation between recording quality and storage capacity for a standard 700 Mb CD recordable. Only Digitalk is not a Microsoft supported compression method. The others all are.

	Recording quality
	CD capacity
	Bit stream

	G.711 A-law
	25 hours
	8000Bs

	G.711 u-law
	25 hours
	8000Bs

	Digitalk
	191 hours
	1062.5Bs

	G.723 High rate
	254 hours
	800Bs

	G.723 Low rate
	307 hours
	662.5Bs


By default the FeaturePhone 175 is set to record in G.723 High rate compression format.

3.6 Fax Recording XE "Fax Recording" 
Fax recording is used for the recording of fax transmissions. A fax machine can be connected to the phone connection next to the line connection. 

As an option the FeaturePhone 175 software can be acquired with the ability to reconvert fax recordings back to the original fax image.

When fax recording is enabled a few recording properties are set automatically. Notification is turned Off because it interferes with the fax. Automatic Gain control is turned Off because it influences the recording volume. Recording quality is set to G.711 A-Law.

Setting Fax recording is described in § 10.3.9.
3.7 Recording from the microphone

Microphone recordings can be started when the telephone is idle.

Press the [image: image20.png]


 Rec. key.

Wednesday 020703 22:46

Record from Microphone

Keep the [image: image21.png]


 Rec. key pressed and press the [image: image22.png]


 Play key to start the recording.

Microphone recording

Wednesday 02-07-03  22:46

The display will indicate that a microphone recording is taking place.

Press the [image: image23.png]


 Stop key to stop the recording.

Code = ▓ _ _ _ _ _ _ _ __

STORE      59      DELETE

Enter a Code to store with the recording

Press STORE to store the recording

The FeaturePhone has three different microphones to record from. See § 10.3.10 for details.

Microphone mode XE "Microphone mode" 
The FeaturePhone 175 has two separate microphones. 

The Handset microphone,

and the speakerphone microphone.

The FeaturePhone 175 has another separate microphone conection that is associated with the option to connect a headset.  It is also possible to connect a table top microphone to the headset microphone port. 

In the settings of the FeaturePhone 175 the different microphones can be selected to record from. This setting has consequences for the microphone recordings and the recording of the welcome and notification messages.

The microphone mode is set in §10.3.10.

3.8 Automatic Gain Control XE "Automatic Gain Control" 
Automatic Gain Control adjusts the recording level of the local and remote voice to attain similar audio levels during playback of the recording. In normal use with a standard telephone line the Automatic Gain Control should not be necessary. Automatic Gain Control is set in §10.3.12
Search XE "Search"  and play recordings

The FeaturePhone 175 has three different kinds of recordings. They are:

· Call recordings,

· Microphone recordings,

· Answering machine messages.

Search and playback of these recordings is initiated with the corresponding function keys. Each recording type has its own Search and playback function key.

[image: image24.bmp] for search and playback of recorded calls

[image: image25.bmp] for search and playback of microphone recordings 

[image: image26.bmp] for search and playback of answering machine messages

The search and playback menus and operation thereof for the different recordings are virtually the same.

Recordings can be searched for according to Date and Code. 

For all recordings the date of the recording is stored with the recording. 

The Code of a recording can be addressed when storing the recording. By default the dialed number or the caller ID is used as Code of a recording. 

When a search is started one has to choose between searching the CD or the memory.  Searching the CD or the memory is done in exactly the same way. The only difference between the two is that recordings can be removed from memory. Search and playback from memory is therefore always accompanied by a menu key ERASE.

During the search the ( character can be used as a wildcard. The wildcard is default used for the Code search string. This way the search is performed on Date and time only. In the next paragraphs the searching for the various recordings is described. 

3.9 Search recordings

Press one of the Playback function keys.

In this example the [image: image27.png]


 Playback Calls is used.

Playback Calls

CD   MEMORY   LAST   STOP

Options:

Press CD to search the CD for the desired recordings

Press MEMORY to search the memory for the desired recordings.

Press LAST to play the last recorded call. (If available)

When CD or MEMORY is chosen the search continues as below.

3.10 Search according to date XE "Search:according to date" 
Following from the previous paragraph the display will show:

Calls from:  ▓8-06-03

SEARCH  CODE         STOP

Enter the date of the call you want to search for.

Press SEARCH to execute the entered query.

The display will show:

18-06-03  09:48      1/23

SEARCH  CODE        STOP

In this example 23 recordings were found. Now the recorder keys can be used to scroll through the recordings and play them. 

Other options are:

SEARCH to start a new search

CODE to expand the search to the code of the recording. The code search is described below.

STOP to quit searching.

3.11 Search according to code XE "Search:according to code" 
In the previous paragraph the search according to date has been described. The search can be expanded to incorporate the code of the recording. Throughout the search the code can be incorporated as follows:

Press CODE when you are in the search menu.

The display will show:

Code =  ▓________________

SEARCH   DATE        STOP

Enter the desired code you want to search for.

Press SEARCH to search for the entered code.

18-06-03  09:48      1/12

SEARCH  CODE         STOP

Now the recorder keys can be used to select and play the found recordings.

Answering machine XE "Answering machine" 
The FeaturePhone 175 has a simple answering machine with a single welcome message. Settings include:

· Number of rings to answer on. This is the amount of rings to pass before the FeaturePhone 175 answers the phone

· Number of rings to always answer. This is the amount of rings to pass before the FeaturePhone 175 answers the phone when the Answering Machine was turned Off.  Hereafter the answering machine stays turned on.

· Enable speaker after answer. This function turns the speaker on when the FeaturePhone 175 answers an incoming call. The playing of the welcome message and the caller leaving a message sounds through the speaker.

· Maximum message time. This function restricts the duration of the messages.

When new messages have been left on the machine the display will indicate so. When messages have been listened to the FeaturePhone 175 will prompt you whether or not to save the messages..For playback of the stored messages see § 6.3. 

Depending on whether recordings are automatically copied to CD the messages will either be stored in memory only, or when Automatic copy to CD mode is active the messages will be copied to CD as well. Once messages have been copied to CD they cannot be removed anymore. When messages are saved in memory messages can be selected and erased individually. 

3.12  Turn the answering machine On

When the FeaturePhone 175 is idle the answering machine can be turned on without entering the settings menu.

Wednesday  18-06-03 15:25

Press the far right menu key. 

Answering Machine

ON    OFF           STOP

Press ON to turn the answering machine On.

To disable the answering machine follow the same instructions as above and press OFF to turn the answering machine Off.

3.13 Record welcome message XE "Welcome message" 
The welcome message is the message the FeaturePhone 175 answers an incoming call with when the answering machine is turned on.

The welcome message is recorded through the handset.

Press Configuration
General Settings

NEXT   MENU          STOP

Press NEXT three times

Answ. Machine Settings

       MENU          STOP

Press MENU

Answ. Machine Settings

Answering Machine:   Off

NEXT         CHANGE  BACK

Press NEXT 

Answ. Machine Settings

Welcome message

NEXT         DELETE  BACK

Press [image: image28.png]


 Rec key to start recording the welcome message.

Press [image: image29.png]


 Stop key to stop the recording

Press [image: image30.png]


 Play key to play the welcome message

To record a new message press the [image: image31.png]


 Rec key again.

To save a recorded welcome message press BACK followed by STOP.

3.14 New Messages XE "Messages" .

When there are new messages on the FeaturePhone 175 the display will say:

Wednesday  18-06-03 16:23

  3 new messages

Press the Playback messages function key to start listening to the new messages. Playback of the new messages will start immediately.

All the new messages have to be listened to before they can be saved. During playback messages can be skipped with the [image: image32.png]


 key. When all the new messages have been played or scrolled through you will be prompted whether to save or delete the messages. 

3.15 Playback XE "Playback"  messages

Press Playback messages function key to start searching for messages.

Playback Messages

CD   MEMORY   LAST   STOP

Press CD or MEMORY depending on where the messages you are looking for are located 

Messages from:  ▓8-06-03

SEARCH  CODE         STOP

Enter the date of the messages or use ( as a wildcard to leave date month or year open.

Press SEARCH to execute the entered query.

or

Press CODE to search according to code.

Code =  ▓________________

SEARCH  DATE         STOP

Press SEARCH to execute the entered query.

18-06-03  09:48      1/23

SEARCH  CODE         STOP

Now the recorder keys can be used to select and play the found messages.

3.16 Remove messages

New messages need to be saved in memory before messages can be selected and removed. This is done as follows:

Perform a search for messages in memory as described in § 6.4.

18-06-03  09:48      1/23

SEARCH  CODE  ERASE  STOP

Then use the recorder keys to select the message that has to be removed.

18-06-03  09:48      9/23

SEARCH  CODE  ERASE  STOP

Press ERASE to remove the message.

When all messages that do not need to be saved are removed the rest of the messages can be saved by copying the memory to CD. See § 7.2 for a detailed description.

4 Operating the CD recorder

The CD recordable has become the standard as carrier for digital data. The following paragraphs describe the operation of the CD functions that are grouped in the disk menu. The disk menu is started with the disk [image: image33.png]


 key . For a detailed description of the features themselves we refer to Chapter 4.

4.1 Free time left XE "Free time left"  on CD

Press the [image: image34.png]


Disk key to enter the disk menu.

The display will show:

Free time on CD = 252:48

NEXT   EJECT        STOP

The number in the display is an indication for the available recording space on the CD. Effectively the available recording space is slightly smaller due to space consumed when a directory is being made. See § 4.3 for details.

4.2 Copy the memory to CD XE "Copy Memory to CD" 
Press the [image: image35.png]


Disk key to enter the disk menu and press NEXT once.

The display will show:

Copy Memory to CD

NEXT  EJECT  START   STOP

Press START to copy the recordings in Memory to CD.

The display will show:

Copy Memory to CD

YES      Start?       NO

Press YES to start copying the recordings.

When the FeaturePhone 175 is finished copying the display will show:

Copy completed

      EJECT          STOP

Press STOP to finish.

After copying the memory to CD the memory is cleared  and the FeaturePhone 175 returns to idle.

4.3 Copy a recording from CD to memory XE "Copy from CD to memory" 
It is also possible to copy a recording from the CD back to memory. 

Select the recording you want to copy to memory. 

Press the [image: image36.png]


Disk key.

The display will show:

Copy from CD to Memory

YES      Start?      NO

Press YES to start copying the recording to memory.

When the FeaturePhone 175 is finished it will show the playback screen where the copied recording was selected but with the next recording selected. Only one recording at the time can be copied to memory. By having the next recording selected one can copy the next recording just by pressing the [image: image37.png]


Disk key again.

4.4 Create CD directory XE "Create CD directory" 
Creating a directory on the CD is done to make the CD readable for the PC as described in § 4.3
Press the [image: image38.png]


Disk key to enter the disk menu and press NEXT twice.

The display will show:

Create CD Directory

NEXT  EJECT  START   STOP

Press START to start creating a directory.

The display will show:

Create CD Directory

OPEN  CLOSE        CANCEL

Press OPEN or CLOSED to create an open or closed directory.

An open directory leaves the remaining space on the CD open for recording. 

A closed directory disables the CD for further recording.

4.5 Play audio CD XE "Play audio CD" 
The FeaturePhone 175 is able to play regular audio CDs.

Press the [image: image39.png]


Disk key to enter the disk menu and press NEXT three times.

The display will show:

Play Audio CD

NEXT  EJECT  START   STOP

Press START to start playing the Audio CD. If no Audio CD is available you will be prompted to insert an Audio CD.

The recorder keys can be used to start/stop, FF and FW or skip the recordings.

Erase CD Re-Writable XE "Erase CD Re-Writable" 
If a CD rewritable is used the recordings on the CD can be erased selectively.  

Press the [image: image40.png]


Disk key to enter the disk menu and press NEXT four times.

The display will show:

Erase CD Re-Writable

NEXT  EJECT  START   STOP

Press START to proceed erasing the CD

The display will show:

Erase CD Re-Writable

QUICK  FULL          BACK

Press QUICK or FULL to proceed 

Choose QUICK to erase the CD quickly

Choose FULL to erase the CD thoroughly

The display will show:

CD Quick Erase 

YES       Start?       NO

Press YES to start erasing.

When the FeaturePhone 175 is finished erasing the display will show:

CD Erase completed

      EJECT          STOP

Press STOP to finish.

4.6 Erase memory XE "Erase memory" 
The memory of the FeaturePhone 175 can be erased as follows:

Press the [image: image41.png]


Disk key to enter the disk menu and press NEXT five times.

The display will show:

Erase memory

NEXT         START   STOP

Press START to proceed erasing the memory.

The display will show:

Erase memory

YES      Start?       NO

Press YES to start erasing the memory.

When the FeaturePhone 175 is finished the display will show:

Memory erase completed.

                    STOP

Press STOP to finish.

4.7 Total recordings XE "Total recordings" 
The FeaturePhone 175 has a few statistical abilities. The total amount of recordings and the oldest recording can be found easily, both for the recordings on CD and for the recordings in memory.

Press the [image: image42.png]


Disk key to enter the disk menu and press NEXT six times.

The display will show:

Total recordings

NEXT   CD    MEMORY  STOP

Press CD or MEMORY to find the total number of recordings in memory or on CD.

The display will show:

Total on CD = 234

NEXT   CD    MEMORY  STOP

Press NEXT to proceed to Oldest recording.

The display will show:

Oldest recording

NEXT   CD    MEMORY  STOP

Press CD or MEMORY to find the oldest recording in memory or on CD.

The display will show:

Oldest on CD = 

NEXT   CD    MEMORY  STOP

Press STOP to finish.

4.8 Software update XE "Software update" 
The FeaturePhone 175 has the ability to update its software from CD.

Press the [image: image43.png]


Disk key to enter the disk menu and press NEXT eight times.

The display will show:

Software update

     EJECT   START   STOP

Place the CD with the software update in the CD drive and press START
It may take a few seconds for the FeaturePhone 175 to recognise the CD.

The display will show:

Software update

YES     Start?       NO

Press YES to start the software update.

While searching for the update the display will show:

Software update

  Busy......

The various software parts will be updated automatically when newer versions have been found. During the updating the display will show:

One moment please!

WARNING! Do not interrupt the FeaturePhone 175 in any way at this point! Interrupting the software update may cause un-restorable failure in the FeaturePhone 175! 

When the FeaturePhone 175 is finished the display will show:

Update finished

                     STOP

Press STOP to finish.

The recordings in memory and your settings will remain intact after the software update.

The Telephone book XE "Telephone book" 
The FeaturePhone 175 has an extensive History and Telephone book function. When Caller ID is configured on your telephone line the FeaturePhone 175 will automatically display the dialing number when the telephone is ringing. Both the numbers of incoming and outgoing calls are stored in the history memory. By default the FeaturePhone 175 also stores the numbers as the code of the recordings.

4.9 Add a contact XE "Add a contact" 
Type in the number that you want to add to the telephone book

123

STORE         CLEAR  STOP

Press STORE to add the number to the telephone book.

Name:                                              STOP

Enter the name you want to store the number with. Consult the character map at the beginning of the manual for more details how to enter the name.

Name: CHARLY   

                                        STORE         CLEAR  STOP 

Press STORE to store the number

CHARLY

123

YES      store?       NO

Press YES to store the entry in the Telephone book.

4.10 Making a call from the telephone book

Press the Telephone book function key [image: image44.bmp].

The display will show:

Telephone book

(press A-Z)

SEARCH         STOP

Press the numerical key closest to the name you want to search for.

To dial the number, pick up the handset and the number will be dialed automatically when the dial tone has been detected.

4.11 Search a contact XE "Search a contact" 
Press the Telephone book function key [image: image45.bmp]
Telephone book

(press A-Z)

SEARCH         STOP

Press SEARCH to start the search in the telephone book. The first entry in the telephone book will be displayed

Allison

456

NEXT  EDIT  DELETE  STOP 

Press NEXT to scroll through the entries of the telephone book.

4.12 Edit a contact XE "Edit a contact" 
Go through the steps above to locate the entry in the telephone book you want to edit.

Allison

456

NEXT  EDIT  DELETE  STOP 

Press EDIT to edit the entry.

An identical display is shown as when adding a number to the telephone book.

4.13 Remove a contact XE "Remove a contact" 
Go through the steps above to locate the entry in the telephone book you want to edit.

Allison

456

NEXT  EDIT  DELETE  STOP 

Press DELETE to remove the entry from the telephone book.

4.14 Call History

Numbers can also be added to the telephone book through the Call history function. When the telephone is On hook the Flash key [image: image46.png]


 has a call history function. Call history keeps track of remote numbers of all your calls. The numbers for outgoing calls are always known. The numbers for incoming calls can only be detected if Caller ID is enabled.

Press [image: image47.png]


 to enter the call history.

The display will show:

History #1  ▓ 15-07 10:35

006538728364

STORE         CLEAR  STOP

The first call in the history table is the last made call.

The arrow indicates an outgoing call.

Date and time was 15-07 at 10:35 hours.

The [image: image48.png]


 and [image: image49.png]


 keys can be used to scroll through the call history.

Press STORE to move the number to the Telephone book.

Follow the instructions as described in § 8.1.

Using the CryptoCard XE "CryptoCard" 
Encrypting recordings is done to prevent unauthorized playback of and listening to recordings. The FeaturePhone 175 uses a CryptoCard to encrypt recordings. The CryptoCard is a chip card that holds the key to the encryption and decryption. Encrypted recordings can only be played back together with the original or a matching CryptoCard. 

The FeaturePhone 175 is equipped with a card reader specifically for this purpose, it is located on the front side of the device. CryptoCards can be obtained from your FeaturePhone 175 supplier. The use of the CryptoCard encryption function is default enabled by inserting a CryptoCard. 

4.15 Insert the CryptoCard into the FeaturePhone 175

Insert the CryptoCard into the FeaturePhone 175 as shown in the figure. 

[image: image50.png]Galiconr

Gliconder





4.16 The CryptoCard and its PIN and PUC code XE "PIN and PUC code" 
The CryptoCard is supplied with a PIN code and a PUC code. The PIN code is a four-digit code and is requested by the FeaturePhone 175 to enable playback of recordings. The PUC code is a twelve-digit code and is used only when the card has been disabled. The PIN code can be changed with the FeaturePhone 175. See § 10.1.3.
A CryptoCard allows for two invalid PIN code entries. After the third invalid Pin code entry the CryptoCard is disabled. The next time the FeaturePhone 175 will ask for the PUC code to enable the card again. The CryptoCard allows for four invalid PUC code entries. After the fifth invalid PUC code entry the FeaturePhone 175 will disable the card definitively. It is advised to keep the PUC code in a safe place.

4.17 Making an encrypted recording XE "Encrypted recording" 
To make encrypted recordings only it is essential that the Use CryptoCard function is enabled in the Recorder Settings menu. Enable the Use CryptoCard function as described in § 10.1.3. With Use CryptoCard enabled it is no longer possible to make normal, unencrypted recordings without disabling the Use CryptoCard function again.

Insert the CryptoCard in the card reader of the FeaturePhone 175. The display will show:

Monday   22-04-02  11:03

card inserted.

When a valid CryptoCard has been inserted all recordings made thereafter will be encrypted as long as the card remains inserted. Removing the card disables recording automatically.

Encrypted recordings are stored as such and can be played only with a matching CryptoCard inserted. Encrypted recordings can also be played with a PC with the FeaturePhone 175 software, however, they cannot be played on the PC without a CryptoCard matching the card used when the recording was made. See the following section for a detailed explanation.

4.18 Playing an encrypted recording

Encrypted recordings can only be played back with the original or a matching CryptoCard. For playback of an encrypted recording on the FeaturePhone 175 this works quite simply. Search and select the desired recording as described in Chapter 5. If playback of the encrypted recording is started the FeaturePhone 175 will automatically ask for the Card code. 

The display will show:

Card code:   ▓
CANCEL

After entering your PIN code the menu key function CONTINUE is added in the display.

Press CONTINUE to start playback of the selected recording.

It is not necessary to enter your card code every time.

Your supplier of the FeaturePhone 175 can supply you with a Card reader for your PC together with the necessary drivers. The FeaturePhone 175 application will show whether recordings are encrypted or not in the "Use of CryptoCard" column. When an encrypted recording is played the FeaturePhone 175 application will prompt you for the insertion of a matching card and the belonging code. After that the recording will play. The FeaturePhone 175 application has the ability to remove the encryption. Encrypted recordings can be exchanged with other PC's just as any other recording. Encrypted recordings can be played on any PC with FeaturePhone 175 application on it and with the original or matching CryptoCard.

5 Settings XE "Settings" 
All the settings start with the Configuration function key followed by the menu keys whose function is determined by the text in the display. As described in Chapter 3.

There are four different categories of settings:

General Settings,

Telephone Settings,

Recorder Settings,

Answering machine Settings.

For every category the settings are made up of a sequence of questions. Most questions can be set to ON or OFF, or a numerical value can be entered or adjusted. The sequence of questions in each category are described in the following paragraphs. If ony a particular question is desired scroll through the menu until you arive at the desired question.

General Settings XE "General Settings" 
The General Settings of the FeaturePhone 175 deal with the basic setup of the device. 

The General Settings cover:

Password protection, see § 10.1.2


CryptoCard, see § 10.1.3
Clock, see § 10.1.4
Language, see § 10.1.5
Reset all Values, see § 10.1.6
5.1.1 Enter General Settings

Monday  03-06-03  10:44

Press [image: image51.png]


Configuration.
General Settings

NEXT  MENU           STOP

Press MENU  to enter the General Settings menu.

Other options are:

NEXT to move to Telephone Settings.

STOP to leave the settings.

General Settings

Password:             Off

NEXT          CHANGE BACK

5.1.2 Password XE "Password"  protection

Password protection is provided as a minimum level of prevention agains unauthorized setting changes. When Password protection is enabled all the settings and play back functions are disabled.

Enable Password protection as follows:

Enter the General Settings menu as in § 10.1.1
General Settings

Password:             Off

NEXT          CHANGE BACK

Press CHANGE to enable Password protection.

Other options are:

NEXT to move to CryptoCard setting.

STOP to leave the General Settings menu.

New Password:   ▓
STORE              CANCEL

Enter a numerical password.

Press STORE.
Repeat Password:   ▓
STORE              CANCEL

Repeat the password.

Press STORE again to store the password.

General Settings

Password:              On

NEXT          CHANGE BACK

Other options are:

NEXT to go to CryptoCard setting.


CHANGE to disable Password protection.
BACK to leave the General Settings menu.

For every time the Password protection is enabled a new password has to be entered. This is to prevent the password protection to be enabled without the password being known.

5.1.3 CryptoCard

The CryptoCard function is used to encrypt recordings. The CryptoCard function is described in Chapter 9.

Enable the CryptoCard function as follows:

Enter the General Settings menu and scroll through the menu until the following display or continue from the previous paragraph by pressing NEXT:

General Settings

CryptoCard:           Off

NEXT          CHANGE BACK

Press CHANGE to enable or disable the CryptoCard function. 

General Settings

CryptoCard:            On

NEXT          CHANGE BACK

Only in case a CryptoCard is inserted the personal code of the card can be set or changed.

Insert your CryptoCard and press NEXT to set the personal code of your card. The display will show:

General Settings

Change Card Code

NEXT          CHANGE BACK

Press CHANGE to change Card code.

The display will show:

General Settings

Card Code: ▓         

STORE              CANCEL

First enter the original code of the card and press CONTINUE.

The display will show:

New Card Code:   ▓
STORE              CANCEL

Enter a numerical code and press STORE.

Repeat Card Code:   ▓
STORE              CANCEL

Repeat the code and press STORE again to store the code.

General Settings

CryptoCard:            On

NEXT          CHANGE BACK

Other options:

CHANGE to disable CryptoCard protection.

NEXT to continue with setting the clock.

BACK to leave General Settings.

5.1.4 Clock XE "Clock" 
Set the clock as follows:

Enter the General Settings menu and scroll through the menu or continue from the previous paragraph by pressing NEXT:

 General Settings 

Clock: Mon 12-06-03 10:31           NEXT          CHANGE BACK

Press CHANGE to set the clock

 General Settings 

Clock: ▓on 12-06-03 10:31           STORE   <<    >>   CANCEL

Press keys 1 - 7 to change the day beginning with 1 for Sunday

 General Settings 

Clock: Mon ▓2-06-03 10:31           STORE   <<    >>   CANCEL

Enter the date (format depending on selected language)

 General Settings 

Clock: Mon 12-06-03 ▓0:31           STORE   <<    >>   CANCEL

Enter the time

 General Settings 

Clock: ▓on 12-06-03 10:31           STORE   <<    >>   CANCEL

Press STORE to save the time.

Other options:


CANCEL to return to previous set time.


<< and >> to move the cursor.

 General Settings

Clock: Mon 12-06-03 10:31           NEXT          CHANGE BACK

Options:


CHANGE to set the clock.


NEXT to continue with language selection.


BACK to leave General Settings.

5.1.5 Language XE "Language" 
Select the language as follows:

Enter the General Settings menu and scroll through the menu or continue from the previous paragraph by pressing NEXT: 

 General Settings

Language:     English  NEXT          CHANGE BACK

Press CHANGE to scroll through the available options.

Together with the Language the date format can change also:

Standard  date format is : DD-MM-YY  

When set to American the format is changed to : MM-DD-YY

Other options:


NEXT to continue with Reset all values.


BACK to leave General Settings.

5.1.6 Reset all values XE "Reset all values" 
The Reset all values prompt is used to revert the FeaturePhone 175 to factory settings.

Warning: Reverting to factory settings may cause personal settings to be lost.

Execute the Reset all values as follows:

Enter the General Settings menu and scroll through the menu or continue from the previous paragraph by pressing NEXT: 

 General Settings

Reset all values:      No                   CHANGE  BACK  

Press CHANGE to enable Reset all values operation.

Press BACK to leave General Settings.

Leaving any of the previously described settings will prompt you with the option to save or cancel the changes made as described in § 3.2. 

Telephone Settings XE "Telephone Settings" 
The Telephone Settings of the FeaturePhone 175 deal with the basic setup of the telephone of the device.

Telephone Settings cover:

Caller ID, see § 10.2.2
Put on hold, see § 10.2.3
Flash time, see § 10.2.4
Wait after dial, see § 10.2.5 

Ring settings, see § 10.2.6


5.1.7 Enter the Telephone Settings

Press [image: image52.png]


Configuration.
General Settings

NEXT  MENU           STOP

Press NEXT  to move to the Telephone Settings menu.

Telephone Settings

NEXT  MENU           STOP

Press MENU  to enter the Telephone Settings menu.

Other options:

NEXT to move to Recorder Settings.

STOP to leave the settings.

Telephone Settings

Caller ID:            Off

NEXT          CHANGE BACK

Caller ID XE "Caller ID" 
When the Telephone Settings are entered the display will show:

Telephone Settings

Caller ID:            Off

NEXT          CHANGE BACK

Press CHANGE to enable or disable the Caller ID by selecting the kind of Caller ID that is enabled on your telephone line or turn caller ID off by selecting Off.

There are four kinds of Caller ID supported by the FeaturePhone 175. They are:

DTMF-ring

FSK-ring

DTMF-pola

FSK-pola

DTMF is used in most European countries; FSK is common in the US. Please note that Caller ID is a service provided by your service provider. Usually this service is not for free and has to be enabled on your line. Without this service the Caller ID cannot work on your FeaturePhone 175. Your service provider can advise you on the kind of Caller ID you need to select.

Other options:


NEXT to continue with the Put on hold function.


BACK to leave Telephone Settings.

5.1.8 Put on hold XE "Put on hold" 
The Put on hold property determines the way the Hook-flash function key is operated. Plaatje R

Enter the Telephone Settings menu and scroll through the menu until the display shows Put on hold, or continue from the previous paragraph by pressing NEXT: 

Telephone Settings

Put on hold:   Hook-Flash

NEXT          CHANGE BACK

The two options are:

Hook-Flash or Earth-Flash
By default Hook-Flash is selected as it is the most common.

Your service provider or PABX supplier can advise on the required setting.

Other options:


NEXT to continue with the Flash time function.


BACK to leave Telephone Settings.

5.1.9 Flash time XE "Flash time" 
The Flash time determines the duration of the flash time of the Put on hold property.

Enter the Telephone Settings menu and scroll through the menu until the display shows Flash time, or continue from the previous paragraph by pressing NEXT: 

Telephone Settings

Flash time:        110 ms

NEXT   - CHANGE +   BACK

Press + or – to increase or decrease the value. Default setting of the Flash time is 110 ms. If this value doesn’t function properly your service provider or PBX manufacturer can advise a proper setting. 

Other options:


NEXT to continue with the Wait after dial function.


BACK to leave Telephone Settings.

5.1.10 Wait after dial XE "Wait after dial" 
The Wait after dial is the time the FeaturePhone 175 waits to start recording after the first number is dialed. Default the Wait after dial is set to 4.0 seconds. A shorter time may cause the FeaturePhone 175 to record the dialing of the number. A longer wait after dial may cause the FeaturePhone 175 not to record the beginning of the conversation.

Enter the Telephone Settings menu and scroll through the menu until the display shows Wait after dial or continue from the previous paragraph by pressing NEXT: 

Telephone Settings

Wait after dial:    4.0s

NEXT   - CHANGE +    BACK

Press + or – to increase or decrease the value.

Other options:


NEXT to continue with the Ring settings.


BACK to leave Telephone Settings.

5.1.11 Ring settings XE "Ring settings" 
The ring settings determine the sound of the telephone ringing.

Ring settings cover three independent ring properties:

Ring Volume

Ring Frequency

Ring Variation

The effects of these settings change the sound of the ring are hard to describe. The user is advised to try a few settings to find a comfortable setting. Changing the ring settings will automatically sound the new setting. 

Press CHANGE in the menu to change the ring properties. Pressing change will sound the new set ring property.

Press BACK and STOP to quit the Telephone Settings menu.

Acknowledge to save the changes made.

Recorder Settings XE "Recorder Settings" 
The Recorder Settings deal with the settings related to the recording properties of the FeaturePhone 175. These are:

Automatic recording, § 10.3.2
Automatic CD Copy, § 10.3.3
Notification, § 10.3.4
Ask Store/Delete, § 10.3.5
Beep after store, § 10.3.6
Minimum call time, § 10.3.7
Recording Quality, § 10.3.8
Fax Recording, § 10.3.9
Microphone mode, § 10.3.10
Microphone amplification, § 10.3.11
Automatic gain control, § 10.3.12
50 Hz humm filter, § 10.3.13
Footswitch, § 10.3.14
5.1.12 Enter the Recording Settings

Press [image: image53.png]


Configuration.
General Settings

NEXT  MENU           STOP

Press NEXT twice to move to the Recorder Settings menu.

Recorder Settings

NEXT  MENU           STOP

Press MENU  to enter the Recorder Settings menu.

Recorder Settings

Automatic recording:  On

NEXT          CHANGE BACK

Other options are:

NEXT to move to Answering machine Settings.

STOP to leave the settings.

5.1.13 Automatic recording

The Automatic recording is described in § 4.1.

Automatic recording can be turned On or Off.

When the Recorder Settings are entered the display will show:

Recorder Settings

Automatic recording:   On

NEXT          CHANGE BACK

Press CHANGE to disable Automatic recording.

Other options:


NEXT to continue with the Automatic CD copy function.


BACK to leave Recorder Settings.

5.1.14 Automatic CD Copy

The Automatic CD copy is described in § 4.2. 

Automatic CD copy can be turned On or Off.

Enter the Recorder Settings menu and scroll through the menu until the display shows Automatic CD copy or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Automatic CD copy:     On

NEXT          CHANGE BACK

Press CHANGE to disable Automatic CD copy.

Other options:


NEXT to continue with the Notification function.


BACK to leave Recorder Settings.

Notification

The Notification is described in § 4.2. 

The Notification options are:

Off

Tone

Message

Tone + Message

Enter the Recorder Settings menu and scroll through the menu until the display shows Notification or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Notification:        Off

NEXT          CHANGE BACK

Press CHANGE to select the desired Notification method.

If the notification Message has been selected the next menu item will be the recording of the message. To start recording press the Rec key. Leaving the menu saves the recording. Press DELETE to remove the recording.

Other options:


NEXT to continue with the Ask Store/Delete function.


BACK to leave Recorder Settings.

5.1.15 Ask Store/Delete XE "Ask Store/Delete" 
The Ask Store/Delete function enables the user to determine after the recording has taken place whether you want to keep the recording or not. If Ask Store/Delete is enabled the FeaturePhone 175 will prompt the user after every recording whether to save or delete the recording. There is a period of 60 seconds where you can decide. After the 60 seconds the recording will automatically be stored.

Enter the Recorder Settings menu and scroll through the menu until the display shows Ask Store/Delete or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Ask Store/Delete :    Off

NEXT          CHANGE BACK

Press CHANGE to enable Ask Store/Delete .

Other options:


NEXT to continue with the Beep after store function.


BACK to leave Recorder Settings.

5.1.16 Beep after store XE "Beep after store" 
The Beep after store function sounds a beep after a recording is stored.

Enter the Recorder Settings menu and scroll through the menu until the display shows Beep after store or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Beep after store :    Off

NEXT          CHANGE BACK

Press CHANGE to enable Beep after store .

Other options:


NEXT to continue with the Minimum call time function.


BACK to leave Recorder Settings.

5.1.17 Minimum call time XE "Minimum call time" 
The Minimum call time is the minimum duration of a recording to be stored. Default the Minimum call time is set to 5 seconds. Recordings shorter then 5 seconds will not be stored.

Enter the Recorder Settings menu and scroll through the menu until the display shows Minimum call time or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Minimum call time:     5s

NEXT   - CHANGE +    BACK

Press + or – to increase or decrease the value.

Other options:


NEXT to continue with the Recording quality function.


BACK to leave Recorder Settings.

5.1.18 Recording Quality

The Recording quality is described in § 4.5. 

The Recording quality options are:

G.711-A

G.711-u

Digitalk

G.723-L

G.723-H

Enter the Recorder Settings menu and scroll through the menu until the display shows Recording quality or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Recordingquality: G.723-H

NEXT          CHANGE BACK

Press CHANGE to select the desired Recording quality.

Other options:


NEXT to continue with the Fax recording function.


BACK to leave Recorder Settings.

5.1.19 Fax Recording

The Fax recording function is described in § 4.6.

Enter the Recorder Settings menu and scroll through the menu until the display shows Fax recording or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Fax recording :       Off

NEXT          CHANGE BACK

Press CHANGE to enable Fax recording.

Other options:


NEXT to continue with the Microphone mode.

BACK to leave Recorder Settings.

5.1.20 Microphone mode

The microphone options are descibed in § 4.7.

The Microphone mode options are:

Internal

External

Handset

Enter the Recorder Settings menu and scroll through the menu until the display shows Microphone mode or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Microphone mode: Internal

NEXT          CHANGE BACK

Press CHANGE to select the desired Microphone mode .

Other options:


NEXT to continue with the Microphone amplification.

BACK to leave Recorder Settings.

5.1.21 Microphone amplification

The Microphone amplification determines the amplification of the microphone signal:

Enter the Recorder Settings menu and scroll through the menu until the display shows Microphone amplification or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Mic. amplification:     4

NEXT          CHANGE BACK

Press CHANGE to adjust the Microphone amplification.

Other options:


NEXT to continue with the Automatic Gain control.

BACK to leave Recorder Settings.

5.1.22 Automatic gain control

The Automatic Gain control function is described in § 4.6.

Enter the Recorder Settings menu and scroll through the menu until the display shows Automatic Gain control or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Auto. Gain control:   Off

NEXT          CHANGE BACK

Press CHANGE to enable Automatic Gain control.

Other options:


NEXT to continue with the 50 Hz humm filter.

BACK to leave Recorder settings.

5.1.23 Humm filter XE "Humm filter" 
The Humm filter can help you get rid of power supply noise on the line. 

Enter the Recorder Settings menu and scroll through the menu until the display shows Humm filter or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Humm filter :       Off

NEXT          CHANGE BACK

Press CHANGE to enable Humm filter.

Other options:


NEXT to continue with the Footswitch.

BACK to leave Recorder settings.

5.1.24 Footswitch XE "Footswitch" 
The Footswitch function allows you to use a footswitch for playback of recordings.

Enter the Recorder Settings menu and scroll through the menu until the display shows Footswitch or continue from the previous paragraph by pressing NEXT: 

Recorder Settings

Footswitch :          Off

              CHANGE BACK

Press CHANGE to enable Footswitch use.

Footswitch is the last item in the Recorder Settings menu.

Press BACK to leave the Recorder Settings menu.

5.2 Answering machine Settings XE "Answering machine Settings" 
The Answering Machine Settings contain all the settings related to the functioning of the Answering machine of the FeaturePhone 175. 

Settings include:

Welcome message, § 10.4.3
Rings to answer on, § 10.4.4
Rings always answer, § 10.4.5
Speaker after answer, § 10.4.6
K-break detection, § 10.4.7
Maximum message time, § 10.4.8
Silence period, § 10.4.9
5.2.1 Enter the Answering machine Settings

Press [image: image54.png]


Configuration
General Settings

NEXT  MENU           STOP

Press NEXT three times to move to the Answering machine Settings menu.

Answ. machine Settings

      MENU           STOP

Press MENU  to enter the Answering machine Settings menu. The Answering machine Settings is the last settings menu.

Other option:

Press STOP to leave the settings

Answ. machine Settings

Answering machine:    Off

NEXT          CHANGE BACK

5.2.2 Turn Answering machine on

The answering machine is turned on as follows:

Enter the Answering machine Settings as described above

Answ. machine Settings

Answering machine:    Off

NEXT          CHANGE BACK

Press CHANGE to enable the Answering machine

Answ. machine Settings

Answering machine:     On

NEXT          CHANGE BACK

Other options:


NEXT to continue with the Welcome message

BACK to leave Answering machine Settings.

Another way to turn on the Answering machine is described in § 6.1
5.2.3 Welcome message

The Welcome message is the message that is sounded when the Answering machine answers the phone. There is no default Welcome message. 

Record a welcome message as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows Welcome message or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

Welcome message

NEXT          DELETE BACK

Press the Rec. key to start recording the welcome message.

Speak from a distance of approximately 50cm. from the FeaturePhone 175.

Press the Play key to play the recorded welcome message.

Press NEXT or BACK to save the welcome message.

Press DELETE to delete the welcome message.

Other options:


NEXT to continue with the Rings to answer on

BACK to leave Answering machine Settings.

5.2.4 Rings to answer on XE "Rings to answer on" 
The Rings to answer on determines the number of rings to pass before the FeaturePhone 175 answers the phone when the Answering machine is turned On.

Set the Rings to answer on as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows Rings to answer or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

Rings to answer on:     4

NEXT   - CHANGE +    BACK

Press - or + to set the number of rings to answer on.

Other options:


NEXT to continue with the Rings always answer function


BACK to leave Answering machine Settings.

Rings always answer XE "Rings always answer" 
The Rings always answer determines the number of rings to pass before the FeaturePhone 175 answers the phone when the Answering machine is turned Off.

Set the Rings always answer as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows Rings always answer or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

Rings always answer:   16      

NEXT   - CHANGE +    BACK

Press - or + to set the number of rings to always answer on.

The Answering machine will be turned On automatically after the FeaturePhone 175 has answered the phone once. 

Select Off in case you don't want the FeaturePhone 175 to answer the phone in case the Answering machine is off.

Other options:


NEXT to continue with the Speaker after answer function


BACK to leave Answering machine Settings.

5.2.5 Speaker after answer XE "Speaker after answer" 
The Speaker after answer function determines whether the speaker of the FeaturePhone 175 is turned on when the phone is answered by the answering machine.

Set the Speaker after answer function as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows Speaker after answer or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

Speaker after answer: Off      

NEXT         CHANGE  BACK

Press CHANGE to enable the Speaker after answer function.

Other options:


NEXT to continue with the K-break detection function


BACK to leave Answering machine Settings.

5.2.6 K-break detection XE "K-break detection" 
The K-break detection function only appears in the menu when the Language is set to English. K-break detection is only available in the UK and serves to indicate the remote caller has terminated the connection. When K-break detection is enabled the FeaturePhone 175 stops recording a message immediately after the caller has terminated the connection. 

Set the K-break detection function as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows K-break detection or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

K-break detection:    Off      

NEXT         CHANGE  BACK

Press CHANGE to enable the K-break detection function.

Other options:


NEXT to continue with the Max. message time function


BACK to leave Answering machine Settings.

5.2.7 Maximum message time XE "Maximum message time" 
The Maximum message time determines the maximum duration of a message. Default the Max. message time is set to 120 seconds.

Set the Max. message time as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows Max. message time or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

Max. message time:   120s      

NEXT   - CHANGE +    BACK

Press - or + to set the Maximum message time .

Other options:


NEXT to continue with the Silence period 

BACK to leave Answering machine Settings.

5.2.8 Silence period XE "Silence period" 
The Silence period is a backup to failing busy tone detection to stop recording a message. When the line is silent for a period longer then the set Silence period the FeaturePhone 175 will assume the remote caller haa hung up.

Set the Silence period as follows:

Enter the Answering machine Settings menu and scroll through the menu until the display shows Silence period or continue from the previous paragraph by pressing NEXT.

Answ. machine Settings

Max. message time:   120s      

NEXT   - CHANGE +    BACK

Press - or + to set the Silence period.

Silence period is the last item in the Answering machine Settings menu.

Press BACK to leave the Answering machine Settings menu.

6 Using the PC software XE "PC software"  

The FeaturePhone 175 comes with special PC software. The object of the software is to be able to locate and play recordings quickly. Place the FeaturePhone 175 CD in the CD player of your PC and the FeaturePhone 175 Quick CD access System will quickly create a clear list of the recordings on the CD. Recordings are by default sorted according to date and time. Recordings can be selected and played from the CD.

It is also possible to create an archive on your hard disk. Recordings in the archive on your hard disk can also be given a name. All recordings with the same remote number will acquire the same name automatically. 

An extensive query screen allows for the definition of a query to be executed in the current archive. Queries can be made according to date and time, remote number, name, incoming and outgoing calls, answering machine messages and microphone recordings.

6.1 Install XE "Install"  the software

Placing the CD-ROM in your computer starts software installation XE "software installation" . If the installation is not started automatically, locate the file setup.exe on the CD and start it by double-clicking. Follow the instructions on the screen.

6.2 Start

Click “Start XE "Start" “ in the lower left corner on your desktop. Go to Programs/Vidicode and click on “FeaturePhone 175”. The application will start right away XE "Wizard" 

 XE "Set-up Wizard" . 

The main dialog looks like this:

[image: image55.png]FeaturePhone 175 Quick CD Access System
Eile Action Query Options _Information

[«

= __F
— I

A e on CD

Date & Time | Duration Name Number or Code | Compression
0 14269 o003 @ 03 6723 High
T8 12 wwne @ 20308 6.723 High
7703 1428112 wmn B 0@ 20309, 6723 High
7703 142746 [ 20309, 6723 High
7703 142745 wos B @ 20309, 6723 High
7703 14:27:12 wwxn B 0@ 20309, 6723 High
wonn B * 203090000065609_ 6,723 High
wny @ @ 20309, 6723 High
wny @ @ 20309, 6723 High
woEs2 O B 6723 High
03 123212 wnezn ® @ 00317472163___ G723 High
03123212 wnezn ® @ 00317472163___ G723 High
03 12:13:04 woss @ B 6723 High
03 12:13:04 woss @ B 6723 High
03 1201112 wneE B @ 0044149477243 G723 High
03 114518 wsmw @ B 6723 High
03 10:46:10 [T 3510666, 6723 High
03 1023:38 o B O® 3510665, 6723 High

16 recordings | Recarder ID: 00 | First 6-7-03 10:23:36 | Last 7-7-03 14:26:36 | Total 0 hours 24 mins





6.3 View recordings on CD

Start the FeaturePhone 175 software.

Place a FeaturePhone 175 CD into the CD drive.

Select File in the menu

Select Archive on CD
Select Show the archive on CD
The [image: image56.png]


 key is a shortcut to the above described function.

The software will now start to read the recordings on the CD and create a list.

Every row in the list represents a recording on the CD. The recording properties are displayed in the various columns.

Recording properties are:

Date and Time,

Duration,

Source e.g. Call, message or microphone recording,

Incoming or outgoing,

Name, 

Telephone number or Code,

CryptoCard number,

Compression Technique used,

and the filename.

At the bottom of the application dialog other general properties of the CD are displayed like:

Number of recordings on the CD,

The Recorder ID

Date and time of the first recording,

Date and time of the last recording,

and the total recording time on the CD.

To locate an archive in another location choose: File=>Archive on CD=>Path for the archive on CD.

6.4 Create an archive XE "Archive"  on Hard disk XE "Hard disk"  

To create an archive on your hard disk you need to view an archive on CD first as described above.

Select the recordings you want to transfer to your hard disk.

Select Action in the menu,

Select Transfer to PC,

Select Selected Records.

The [image: image57.png]


 key is a shortcut to the above described function.

Your selection of records will now be copied to the archive on the hard disk. Default the hard disk archive is located in the Archive folder in the directory where the program was installed. Default the program is installed in C:\Program Files\Vidicode\FeaturePhone 175

To view an Archive on HD:

Select File in the menu,

Select Archive on hard disk.

Select Show the current archive on hard disk.

The [image: image58.png]


 key is a shortcut to the above described function.

The application will now make a list of the recordings in the default archive on your hard disk. To locate a different archive on your hard disk:

Select File in the menu,

Select Archive on hard disk,

Select Locate another archive on hard disk.

There are two tool buttons to show the archives on CD and on your hard disk:

[image: image59.png]


 to show the archive on CD and

[image: image60.png]


 to show the archive on hard disk

6.5 Recycle bin XE "Recycle bin" 
Once an archive on hard disk has been created it is also possible to remove recordings from that archive. 

Select the recordings you want to remove from the archive,

Select Action in the menu,

Select Delete Selection.

The [image: image61.png]


 key is a shortcut to the above described function.

The selection of recordings in now moved to the recycle bin.

The recycle bin works like the standard Windows Recycle Bin. Recording removed from an archive are placed in the recycle bin. Recordings can be restored from the recycle bin. Recordings removed from the recycle bin are lost.

To view recordings in the recycle bin:

Select File in the menu,

Select Recycle bin. 

The program will now generate a list of recordings in the recycle bin.

The [image: image62.png]


 key is a shortcut to the above described function.

In the recycle bin recordings can be selected to be removed or restored. Both options are available in the Action menu.

6.6 Export XE "Export"  recordings

Recordings can be exported from the application.

Select the recording you want to export,

Select Action in the menu,

Select Export.

You now have the option to export the file in the same format as the recording is or to export the recording as a WAV file.

6.7 On Line software update XE "On Line software update"  

The FeaturePhone 175 Quick CD access System has the ability to perform an automatic on Line update. You need an open internet connection for this feature to work.

Select Information in the menu

Select Update product on Line

The following dialog appears:

[image: image63.png]Update this product to the latest version [-[o[x]

Cunentversion, 100.0013
f Check for updates

Ex2site Update

Exit





Click Check for updates to check for available updates.

The available update will be listed.

Click Execute Update to install the update. Keep in mind that an update may be of considerable size which can take a while to download.

6.8 Search recordings

The FeaturePhone 175 software has an extensive query screen to search and select recordings.

The query screen looks like this:

[image: image64.png]FeaturePhone 175 Quick CD Access System
Eile Action Query Options _Information

Oom

Define Your Query
€ This Year

«|  januari 281 ) o

€ Last Year
e

Year Before Last e s
€ This Month 6 7 8 s M |7 s@w0 MR
€ Last Month 131416 16 17 18 19 | (14 15 16 17 18 19 20
 Month Before Last 202 2 2 2002 2% | |2 223256
® T 2w w %2 3w
€ Last Week
C Week Before Last

I Telephone Calls
& Use Calendar ¥ Answering Machine Messages ¥ Incoming Calls
coail I Microphone Recordings ¥ Outgoing Calls
Name

Cancel Queries
Cancel
oK

Telephone Number or Code
Duration in Seconds (e.g. 30-300)

18 recardings | Recorder ID: 00 | First 6-7-03 10:23:38 | Last 7-7-03 14:28:36 | Total: 0 haurs 24 mins





To the left a group of radio buttons can be selected to view recordings from the specified interval.

Check Use Calender to select a date range within the calenders. 

Select the starting date in the left calender 

Select the end date in the right calender.

Click OK to execute the selected query.

6.9 Software Settings XE "Software Settings" 
The settings of the software are located in a separate dialog.

Settings are devided over 5 tab pages as you can see in the following dialog:

[image: image65.png]Program options

[ Fax Conversion ) Securty Language
CrypiaCard Database
Fields in Database G Preferences
W Date_and Time Font
© Durafion
© Saurce ™ Can Change Raw Height
P ouf
HEERIAIE I L A I Can Change Column Order
® Name
¥ Telephonenumber ar Cade
I™ CiypiaCard Number
P Compressian Technicue Used
I Filename
Cancel Apply





Selected changes are executed when the options screen is left clicking Apply. Click Cancel to return to the main dialog retaining the original settings.

In the Database tab the various recording properties columns can be selected to show in the main dialog. 

Click Font to change the font of the list of recordings.

Other settings include:

Fax conversion settings in the Fax Conversion tab

CryptoCard settings in the CryptoCard tab

Password protection in the Security tab

The Language tab will allow you to select a desired language.

For a detailed description of the settings we refer to the helpfiles of the program. These are always in compliance with the software version and the available settings.

Accessories

There are several accessories XE "accessories"  available for your Call Recorder:

Call Recorder Archive software for Microsoft Windows XE "Archive software for Windows"  (versions 95/98/ME/XP/2000/NT4), order number 932

The function of this software is to archive the recordings of your Call Recorders on a PC. These recordings can be played directly from the archive via the loudspeakers of the PC.

This software can be used with all model Call Recorders that possess a network interface. It is therefore possible to build up a combined archive of recordings made with Call Recorders ISDN and Call Recorders for analogue lines.    

The software approaches Call Recorders as FTP servers. This method of connecting can be used in almost every type of network connection. For more information see Chapter Error! Reference source not found..

The most important functions of the software are:   

Retrieving the contents of 1 to 4 recorders and displaying them in a database   

Searching for recordings in an often very extensive database with the help of queries (search commands)    

Transferring all recordings or a selection of them to the hard disk of the PC  

Playing via the loudspeakers of a PC      

Exporting to standard PC format for sending to a third party via e-mail or another medium 

Making sub-archives for storage on CD or DVD with the help of a CD recorder or DVD recorder       

The Archive Software can also be used to automatically retrieve all recordings, so that there is always a back up of them present in the recorders. The Archive Software is an almost indispensable accessory of the Call Recorder ISDN.         

Crypto Card option XE "Crypto Card option" , order numbers 930 and 931

This option has been extensively described in chapter Error! Reference source not found.. 

Crypto Cards can be supplied in two different ways:

Part 930, set of 3 identical cards

Part 931, set of 4 x 3 different normal cards and 3 master cards that can be used to play the recordings of the other cards.

Article 931 is meant to be used as security in combination with Call Recorder Archive Software that can archive the recordings of four recorders in a PC via LAN, Internet or intranet.

Monitor Software XE "Monitor Software" , order number 933

Monitor Software makes it possible to listen in to the recordings of a Call Recorder via a LAN or via Internet. The user can choose from a list of recorders.

Crypto Card reader for the PC XE "Crypto Card reader PC" , order number 934

The Crypto Card reader is needed for playing recordings made with a Crypto Card on a PC. It can be used both with Call Recorder Archive Software and with Monitor Software.

Table top Microphone for Recording Meetings XE "microphone for recording meetings" , order number 922

When the Call Recorder is regularly used for recording meetings, some users prefer a tabletop microphone to use of the headset microphone. Various common models can be used. The microphone referred to here is one especially designed for video conferencing, eminently suitable for recording group conferences.

Multimedia Loudspeakers, order number 218

When the internal loudspeakers do not suffice, multi media loudspeakers with built-in amplifiers can be connected to the LS-connector of the recorder.

7 Conformation with directive 1999/5/EC and Class 1 of the R&TTE

Hereby Retell declares that this telephone conversation recorder is in compliance with the essential requirements and other relevant provisions of directive 1999/5/EC.

Retell declares that your telephone conversation recorder meets the requirements of Class 1 of the R&TTE directive and so is designed as not to adversely affect the operation of the telecommunication network.

A full copy of the certificate is available from the address below

Retell Ltd

53 Thames Street

Sunbury on Thames

Middlesex

TW16 5QH

England

8 Guarantee Conditions XE "Guarantee Conditions" 
This chapter gives a general overview of the guarantee conditions. We can send you a copy of the complete text on request.

Your Call Recorder has a 12-month factory guarantee. The guarantee is effective for normal use only. We would like to emphasize that the guarantee is not valid under exceptional environmental conditions, such as extreme temperatures or humidity levels, nor in the unlikely event of a lightning strike. The guarantee is also not valid if the machine has not been handled properly, for example when it has been dropped, or bumped into. Hard disks are fragile.  

In order to qualify for guarantee, you should contact your supplier, and show the receipt. If your supplier cannot help you, you should contact the manufacturer. The manufacturer reserves the right to determine the final date of the guarantee period on the basis of the date of production. Costs of transport to and from the supplier or the manufacturer are for the buyer’s account.

We would like to stress that the guarantee is for parts only, and does not cover any costs resulting from the breakdown of the Call Recorder or its software.

Subject to the conditions listed below Retell will repair or at its option replace the equipment supplied by them which in Retell's opinion is faulty or below standard as a result of inferior workmanship or materials. This does not affect your statutory rights.

Applies to issues raised within one year of purchase. Please keep your invoice to prove date of purchase.

Does not cover any faults or defects caused by accident, misuse, fair wear and tear, neglect, tampering with the equipment or any attempt at adjustment or repair except by Retell.

Retell expressly excludes to the extent permitted by law liability for consequential loss, loss of recordings or data, consequences of any unsatisfactory data backup or security, damage or injury arising from or in connection with any of its products.

Retell gives no guarantee that the product will be suitable for the user's purposes or that the product is compatible with the user's telephone system or lines. Retell's maximum remedy for any and all issues is the refund of the cost of the product up to the normal retail price.

9 Legality of Recording

Ofcom says that you should make every reasonable effort to inform all parties to a call that it may or will be recorded, silently monitored or intruded into. They say that it is up to you how you do this and that acceptable options, depending on circumstances, might include warning tones, pre-recorded messages, spoken warnings by the operator or written warnings included in publicity material, telephone directories, contracts, terms of business, staff notices etc. Ofcom says that it may not always be possible to warn first time callers with whom you have had no previous contact but what is important is that you have a systematic procedure in place, which provides the necessary information where this is a realistic possibility. Ofcom also suggest following on a ruling from the European Court of Human Rights that staff are told of a phone that they can call from which will not be recorded. 

If you require further information or for the latest updates since this leaflet was printed contact www.ofcom.org.uk or contact them direct at: Phone: 0845 456 3000 or 020 7981 3040. Email:contact@ofcom.org.uk, Fax: 0845 456 3333 
Post: Ofcom Contact Centre, Riverside House, 2a Southwark Bridge Road
London, SE1 9HA

(Note that Ofcom has taken over Ofcom's responsibilities)

Check www.retellrecorders .co.uk for information on recording staff or individuals.

Index



A
Accept a call, 18

accessories, 68

Add a contact, 38

Answering machine, 28

Answering machine Settings, 57

Archive, 63

Archive software for Windows, 68

Ask Store/Delete, 54

Automatic Gain Control, 23

Automatic recording, 21

B
Beep after store, 54

C
Caller ID, 50

Clock, 14, 47

connections, 12

Copy from CD to memory, 31

Copy Memory to CD, 31

Copy recordings to CD, 21

Create CD directory, 32

Crypto Card option, 68

Crypto Card reader PC, 68

CryptoCard, 41

E
Edit a contact, 39

Encrypted recording, 41

End a call, 19

Erase CD Re-Writable, 34

Erase memory, 34

Export, 64

F
Fax Recording, 22

Flash time, 50

Footswitch, 57

Free time left, 31

Function keys, 20

G
General Settings, 45

Guarantee Conditions, 70

H
Handset, 12

Hard disk, 63

Headset, 17

Humm filter, 56

I
Initialise CD, 18

Install, 62

K
K-break detection, 60

Key indicators, 20

L
Language, 15, 47

M
Making a CD, 21

Maximum message time, 61

Menu keys, 20

Messages, 29

microphone for recording meetings, 68

Microphone mode, 23

Minimum call time, 55

Monitor Software, 68

N
Notification, 21

O
On Line software update, 64

P
Password, 45

PC software, 62

PIN and PUC code, 41

Play audio CD, 33

Playback, 29

Power adapter, 14

Put on hold, 50

R
Recorder Settings, 52

Recording calls, 21

Recording quality, 22

Recycle bin, 63

Remove a contact, 39

Reset all values, 48

Ring settings, 51

Rings always answer, 60

Rings to answer on, 58

S
Search, 25

according to code, 26

according to date, 25

Search a contact, 38

Settings, 44

Set-up Wizard, 62

Silence period, 61

software installation, 62

Software Settings, 65

Software update, 35

Speaker after answer, 60

Speakerphone, 17

Start, 62

Start a call, 18

T
Telephone book, 38

Telephone line, 14

Telephone Settings, 49

Total recordings, 35

U
User interface, 20

V
volume, 19

W
Wait after dial, 51

Welcome message, 28

Wizard, 62



[image: image67.jpg]9 5=
1”51\1





Retell Ltd 


53 Thames Street,


Sunbury on Thames


Middlesex


TW16 5QH


England


Tel: +44 (0)1932 779755


Fax: +44 (0)1932 780383


Email: � HYPERLINK "mailto:sales@retell.co.uk" ��sales@retell.co.uk�


www.retellrecorders.co.uk


© Retell 2004











Manual FeaturePhone 175  © Retell Ltd 2003

Need help? Call Retell on 0871 42 42 444

3
Manual FeaturePhone 175  © Retell Ltd 2003 

Need help? Call Retell on 0871 42 42 444


